ZhangjiagangYarui Chemical Co.,Ltd.

MATERIAL SAFETY DATA SHEET

TRANSPORTATION EMERGENCY

RM307B, BLDG. 10, Orient New Space, Yangshe Town,

Tel: 86-512-58961068

Fax: 86-512-58961066

Email:sales@yaruichem.com

Issue Date: June 20, 2010

1. Product and Company Identification

Zhangjiagang City, Jiangsu Province 215600, China

Product Name:

:

2. Hazards Identification

Diethyltoluenediamine (DETDA)

Emergency Overview

Warning	Color: Light, tan to brown   Form: liquid    Odor: Slight, Amine.

Harmful by inhalation, in contact with skin and if swallowed.  Inhalation, skin absorption, or ingestion may

cause methemoglobin formation resulting in a reduced ability of the blood to carry oxygen; a symptom of

this may be cyanosis (purplish-blue coloring of skin, fingernails, and lips).  Toxic gases/fumes may be

given off during burning or thermal decomposition.  Use cold water spray to cool fire-exposed containers to

minimize the risk of rupture.  May cause respiratory tract irritation.  May cause skin irritation.  Causes eye

irritation.  May cause liver damage.

Potential Health Effects

Primary Routes of Entry:

Medical Conditions Aggravated by

Exposure:

Skin Contact, Eye Contact

Eye disorders, Respiratory disorders, Skin disorders

HUMAN EFFECTS AND SYMPTOMS OF OVEREXPOSURE

Inhalation

Acute Inhalation

May cause methemoglobin formation resulting in a reduced ability of the blood to carry oxygen; a

symptom of this may be cyanosis (purplish-blue coloring of the skin, fingernails, and lips). Inhalation is

unlikely due to the low vapor pressure.  If misted or handled at elevated temperatures, high concentrations

may cause respiratory tract irritation.

Chronic Inhalation

May cause liver damage.

Page: 1 of 8    Report version: 1.6

Skin

Acute Skin

May cause methemoglobin formation resulting in a reduced ability of the blood to carry oxygen; a

symptom of this may be cyanosis (purplish-blue coloring of the skin, fingernails, and lips). If sufficient

amounts are absorbed, systemic toxicity may occur with symptoms similar  to those described in acute

inhalation.

Eye

Acute Eye

Causes irritation with symptoms of reddening, tearing, stinging, and swelling.

Ingestion

Acute Ingestion

Symptoms of ingestion may include abdominal pain, nausea, vomiting, and diarrhea. May cause

methemoglobin formation resulting in a reduced ability of the blood to carry oxygen; a symptom of this

may be cyanosis (purplish-blue coloring of the skin, fingernails, and lips). Harmful if swallowed.

Chronic Ingestion

May cause liver damage.

Carcinogenicity:

No Carcinogenic substances as defined by IARC, NTP and/or OSHA

3. Composition/Information on Ingredients

Hazardous components

Weight percent

98%

4. First aid measures

Eye contact

Component

Diethyltoluenediamine (DETDA)

CAS-No.

68479-98-1

In case of contact, flush with plenty of water for at least 15 minutes. Use fingers to ensure that eyelids are

separated and that the eye is being irrigated. Call a physician immediately.

Skin contact

Immediately remove contaminated clothing and shoes. Wash affected areas, including hair, beneath nails

and other concealed areas with Polyethylene Glycol 400.  Repeat the washing with soap and water. If

Polyethylene Glycol 400 is not available, wash immediately with soap and plenty of cold water. Wash

clothing and shoes before reuse. Get medical attention.

Inhalation

If inhaled, remove to fresh air. If not breathing, give artificial respiration using a pocket mask type

resuscitator. If breathing is difficult, give oxygen. In case of blue discoloration (cyanosis) of skin, lips, or

fingernails, give oxygen to breathe. Get medical attention.

Ingestion

Page: 2 of 8    Report version: 1.6

If ingested, do not induce vomiting unless directed to do so by medical personnel. Give two glasses of

water for dilution. Do not give anything by mouth to an unconscious person. Call a physician.

Notes to physician

Immediately give oxygen if victim turns blue (lips, ears, fingernails). Since reversion of methaemoglobin to

haemoglobin occurs spontaneously after termination of exposure, moderate degrees of cyanosis need to be

treated only by supportive measures.

5. Fire-fighting measures

Suitable extinguishing media:

Special Fire Fighting Procedures

Carbon dioxide (CO2), Dry chemical, Foam, water spray for large

fires.

Firefighters should be equipped with self-contained breathing apparatus to protect against potentially toxic

and irritating fumes. Toxic and irritating gases/fumes may be given off during burning or thermal

decomposition. Use cold water spray to cool fire-exposed containers to minimize the risk of rupture.

6. Accidental release measures

Spill and Leak Procedures

Remove all sources of ignition, including flames, heat, and sparks. Ventilate area to remove vapors or dust.

Evacuate and keep unnecessary people out of spill area. Use appropriate personal protective equipment

during clean up. Dike or dam spilled material and control further spillage, if possible. Do not allow spilled

material or wash water to enter sewers, surface waters, or groundwater systems. Large spills should be

contained and pumped into original or similar containers. Cover spill with inert material (e. g., dry sand or

earth) and collect for proper disposal. Wash spill area with soap and water. Collect wash water for

approved disposal. Notify local health and safety authorities and other appropriate agencies if necessary.

7. Handling and storage

Storage temperature:

maximum:

Storage period

18 Months

Handling/Storage Precautions

43 °C (109.4 °F)

Handle in accordance with good industrial hygiene and safety practices. Wash thoroughly after handling.

Keep container closed when not in use. Material is hygroscopic and may absorb small amounts of

atmospheric moisture. If contamination with isocyanates is suspected, do not reseal containers. Do not get

on skin or clothing. Do not get in eyes. Do not breathe vapours or spray mist.

8. Exposure controls/personal protection

Diethyltoluenediamine (DETDA)	(68479-98-1)

Zhangjiagang Yarui Chemical Co.,Ltd.

Page: 3 of 8    Report version: 1.6

Time Weighted Average (TWA): 0.02 ppm

Industrial Hygiene/Ventilation Measures

Use local and general exhaust ventilation to control levels of exposure.

Respiratory protection

The following respirator is recommended if airborne concentrations exceed the appropriate

standard/guideline., NIOSH approved, air-purifying respirator with organic vapor cartridges and N-95

filters, Full face-piece is recommended.

Hand protection

Permeation resistant gloves.

Eye protection

Chemical resistant goggles must be worn.

Skin and body protection

Permeation resistant clothing

Additional Protective Measures

Employees should wash their hands and face before eating, drinking, or using tobacco products. Educate

and train employees in the safe use and handling of this product.

9. Physical and chemical properties

Form:

Color:

Odor:

pH:

Freezing Point:

Boiling point/boiling range:

Flash point:

Vapour pressure:

Specific Gravity:

Solubility in Water:

Bulk density:

Hygroscopicity:

10. Stability and reactivity

Hazardous Reactions

liquid

Light, tan to brown

Slight, Amine

Not Established

Not Established

not established

> 93.34 °C (200.01 °F)

Not Established

1.03  @ 25 °C (77 °F)

completely soluble

1,031 kg/m3

hygroscopic

Hazardous polymerisation does not occur.

Stability

Stable

Materials to avoid

Oxidizing agents, Isocyanates

Hazardous decomposition products

By Fire:  Carbon Dioxide; Carbon Monoxide;  Hydrogen cyanide, nitrogen oxides (NOx), Amines, other

aliphatic fragments which have not been determined

Page: 4 of 8    Report version: 1.6

11. Toxicological information

Acute oral toxicity

LD50: 472 mg/kg (Rat, Female)

LD50: 542 mg/kg (rat, male)

Acute inhalation toxicity

LC50: > 2.45 mg/l, 1 h (Rat)

Acute dermal toxicity

LD50: > 1,000 mg/kg (rabbit)

Skin irritation

rabbit, Non-irritating

Eye irritation

rabbit, irritating

Sensitisation

Maximisation Test: non-sensitizer (guinea pig)

Repeated dose toxicity

21 Days, dermal: NOAEL: 1 mg/kg, (rabbit, Male/Female, daily)

13 Days, inhalation: NOAEL: < 10 mg/l, (Rat, )

Mutagenicity

Genetic Toxicity in Vitro:

Ames: positive (Salmonella typhimurium, Metabolic Activation: with)

Positive and negative results were seen in various in vitro studies.

Genetic Toxicity in Vivo:

Dominant Lethal Assay: (rat, Male/Female, oral)

negative

Cytogenetic assay: (Rat, male, oral)

positive

Micronucleus Assay: (mouse, Male/Female, intraperitoneal)

negative

Carcinogenicity

Rat, Male/Female, oral, 2 years, daily,

positive

12. Ecological information

Biodegradation

aerobic, 0 %, Exposure time: 28 Days

Chemical Oxygen Demand (COD)

2,370 mg/g

Acute and Prolonged Toxicity to Fish

LC50: ca. 194 mg/l (Golden orfe (Leuciscus idus), 48 h)

Page: 5 of 8    Report version: 1.6

Acute Toxicity to Aquatic Invertebrates

EC50: ca. 0.5 mg/l (Water flea (Daphnia magna), 48 h)

Toxicity to Microorganisms

EC10: 170 mg/l, (Pseudomonas putida, 24 h)

13. Disposal considerations

Waste Disposal Method

Waste disposal should be in accordance with existing federal, state and local environmental control laws.

Empty Container Precautions

Recondition or dispose of empty container in accordance with governmental regulations. Do not heat or cut

container with electric or gas torch. Empty containers retain product residue (dust, liquid, vapor and/or

gases) and can be dangerous.

14. Transport information

Land transport (DOT)

Non-Regulated

Sea transport (IMDG)

Non-Regulated

Air transport (ICAO/IATA)

Non-Regulated

15. Regulatory information

United States Federal Regulations

OSHA Hazcom Standard Rating:	Hazardous

US. Toxic Substances Control Act:	Listed on the TSCA Inventory.

US. EPA CERCLA Hazardous Substances (40 CFR 302):

Components

None

SARA Section 311/312 Hazard Categories:

Acute Health Hazard, Chronic Health Hazard

US. EPA Emergency Planning and Community Right-To-Know Act (EPCRA) SARA Title III

Section 302 Extremely Hazardous Substance (40 CFR 355, Appendix A):

Components

None

US. EPA Emergency Planning and Community Right-To-Know Act (EPCRA) SARA Title III

Section 313 Toxic Chemicals (40 CFR 372.65) - Supplier Notification Required:

Page: 6 of 8    Report version: 1.6

Components

None

US. EPA Resource Conservation and Recovery Act (RCRA) Composite List of Hazardous Wastes

and Appendix VIII Hazardous Constituents (40 CFR 261):

Under RCRA, it is the responsibility of the person who generates a solid waste, as defined in 40 CFR

261.2, to determine if that waste is a hazardous waste.

State Right-To-Know Information

The following chemicals are specifically listed by individual states; other product specific health and safety

data in other sections of the MSDS may also be applicable for state requirements.  For details on your

regulatory requirements you should contact the appropriate agency in your state.

California Prop. 65:

o

To the best of our knowledge, this product does not contain any of the listed chemicals, which has found to cause cancer,

birth defects or other reproductive harm.

16. Other information

NFPA 704M Rating

Health

Flammability

Reactivity

Other

2

1

0

0=Insignificant  1=Slight  2=Moderate  3=High  4=Extreme

HMIS Rating

Health

Flammability

Physical Hazard

2*

1

0

0=Minimal  1=Slight  2=Moderate  3=Serious  4=Severe

* = Chronic Health Hazard

The method of hazard communication for Jiangsu Changyu Chemical Co.,Ltd. is comprised of Product Labels and

Material Safety Data Sheets.  HMIS and NFPA ratings are provided by Jiangsu Changyu Chemical Co.,Ltd. as a

customer service.

Contact person:

Telephone:

MSDS Number:

Version Date:

Report version:

Product Safety Department

86-512-58668811

112000032092

01/15/2010

1.6

Page: 7 of 8    Report version: 1.6

This information is furnished without warranty, express or implied.  This information is believed to be

accurate to the best knowledge of  Zhangjiagang Yarui Chemical Co.,Ltd. The information in this MSDS relates only

to the specific material designated herein.  Zhangjiagang Yarui Chemical Co.,Ltd. assumes no legal responsibility for

use of or reliance upon the information in this MSDS.

Page: 8 of 8    Report version: 1.6